Design Department Diamond Map

Design Software Details

Design Gold operates as a specialized plugin integrated seamlessly within the Rhinoceros (Rhino) 3D modelling software. This integration provides designers with advanced tools specifically tailored for jewellery design, while utilizing Rhino’s robust modelling capabilities. A key feature of Design Gold is its diamond mapping functionality, which is derived from Jewel CAD design conversion.

Detailed Procedure for Creating Jewel CAD Designs to Gold using Rhino and Design Gold Plugin:

This procedure describes the complete step-by-step process for converting a Jewel CAD design file into a format suitable for gold production using Rhino along with the Design Gold plugin. Before starting, ensure that Jewel CAD, Rhino, and the Design Gold plugin are properly installed and that the plugin is enabled within Rhino

Step 1: Launching Rhino and Accessing the Design Gold Plugin

Open the Rhino 3D modeling software. Once Rhino has fully loaded, confirm that the Design Gold plugin is available. The plugin typically appears as an additional menu or toolbar at the top of the Rhino interface.
If the Design Gold menu is not visible, verify that the plugin has been correctly installed and activated through Rhino’s Plugin Manager.

Step 2: Transferring the Design Object from JewelCAD to Rhino

Click File → New and select the template Small Objects – Millimeters. This ensures the design is opened in millimeter units for accurate formatting.
Next, go to File → Import and select the appropriate .jcd file. The Read JCD File Settings window will appear. Keep all settings at their default values and click OK to proceed.

Step 3: Accessing and Displaying Gem Size Information

Once the file is imported, the design will open in four different views: Top, Front, Right, and Perspective.
Click on the Top view, then right-click on the screen and select the option to convert the wireframe into a solid surface.

Navigate to the Design Gold menu and locate the “Gem Size List” icon. Clicking this icon will open a pop-up window displaying gemstone-related information. Within this pop-up, click the “Show Size” button (or the equivalent option if not already enabled).

This action will display the size directly on each diamond in the Rhino viewport, allowing visual verification of gem data. Diamonds will appear in different colors based on their sizes.
The active view does not affect the result, as the software captures all diamonds in the design, regardless of which view they are visible in.

Step 4: Capturing a Snapshot of the Diamond Map and Information

With the Top view visible and the Gem Size List pop-up open on the screen, take a screenshot using Windows + Print Screen. The screenshot will be automatically saved as a .png image file.

The saved image is programmatically cropped using a Python script to extract a clear and focused view of the diamond map and gem size information.


Flow Design: